Laboratory OROCHI Simulator (LOROS) Guide

Roger Stabbins

23/1/2024

# Introduction:

LOROS can be controlled with ImageCapture or with the Python library ‘orochi_sim_ctrl’ or ‘osc’.

This guide describes how to control LOROS with the Python library.

It also describes how to analyse the data using the accompanying ‘orochi_sim_proc’ or ‘osp’ Python library.

# Software:

Visual Studio Code – for accessing osc and osp, and for accompanying python notebooks.

Github – must be installed to synchronise the versions of osc and osp across multiple computers, and to handle previous versions of osc and osp.

Conda – is used to handle the software environment and dependencies.

TISGrabber – Camera manufacturer software for interfacing with the cameras via Python. See: https://www.theimagingsource.com/en-us/documentation/icpython/

## Note on ‘OROCHI_SIMULATOR_CONTROL’ Directory structure

All software, raw data and processed data is stored in the ‘orochi_simulator_control’ directory (at /kameken/rstabbins/).

Content:

- data			stores data captured during LOROS experiments

- notebooks		legacy directory – ipy-Notebook of prev. osc/osp versions

- sessions		stores experiment notebooks, and processing results

- src			Python osc and osp source code

- testdata		Legacy test data

- tisgrabber		software for camera interfacing with Python

- .gitignore		git-ignore file

- environment.yml	conda environment details for rebuilding the software

Explanation:

- The osc and osp software is stored in src.

- The sessions directory contains subdirectories, where each subdirectory is a ‘session’

- Each ‘session’ will have a ‘capture’ notebook and a ‘processing’ notebook.

- The ‘capture’ notebook interfaces with osc, and logs the setting of the cameras and the capture of the data.

- Data recorded by the ‘capture’ notebook is stored in ‘data’, in a data/sessions/[session] directory

- In the data/sessions/[session] directory, each subdirectory is a ‘scene’ e.g. ‘spectralon’, ‘spectralon_dark_frames’, ‘sample1’, ‘sample1_dark_frames’ etc.

- In each data/sessions/[session]/[scene] directory are the subdirectories for each ‘channel’.

  - A ‘channel’ is defined by the ‘camera number’ as written on the physical camera unit on LOROS, and the centre-wavelength of the camera for the experiment e.g. ‘0_850’.

- Also in the [scene] directory is a ‘camera_config.csv’ file that records the camera serial numbers, spectral information, and importantly the ROI coordinates used for setting the auto-exposure. There is also an ‘exposures_seconds.txt’ file that gives the exposure time for each channel.

- Each [scene]/[channel] directory hosts the raw captured images. The filename format is: [camera number]_[centre-wavelength]_[index]_[image-type].tif

  - index gives the index of the image for cases where repeat images have been captured for averaging

  - image-type can be ‘img’ for a raw image, ‘drk’ for a dark frame, and ‘ave’/’std’ if the images have been averaged before saving, giving the average and standard deviation values over the repeats.

- Metadata such as exposure time, ROI coordinates, camera number etc. are stored in the tif file metadata, that can be read with osp.

- Data is processed with the ‘processing’ notebook by interfacing with osp.

- The processing notebook accesses the data in a ‘./data/sessions/[session]/[scene]’ directory, and puts processed data into the ‘./sessions/[session]/[products]/[scene]’ directory.

- Note, *all directory creation is handled automatically by osc and osp – no directories need to be manually created*.

- The processing notebook may also need to access calibration data, such as flat-fields and geometric calibration matrices. These are accessed from the ‘./data/calibration/[calibration_session]’ directory. Multiple calibration directories exist representing different configurations of LOROS, so this can be specified in the processing notebook.

- The ‘./sessions/[session]/[products]’ directory stores all processed data, keeping it separate from the raw experimental data kept in the ‘./data/sessions/[session]/’ directory.

# Setup:

- Open Visual Studio Code

- Open Folder, choose ‘OROCHI_SIMULATOR_CONTROL’ (full address: kameken/rstabbins/orochi_simulator_control/)

- Navigate to ‘./sessions/’

- Either, choose an existing guide session, or create a new directory. Here we advise duplicating ‘rikkyo_check_imaging_18012024’, and opening the copy to use as a guide.

- Open the notebook, ‘rikkyo_check_imaging_capture_18012024.ipynb’

- Then, follow through the notebook stages to see how osc can be used to control the LOROS:

  - Setup -  explains what variables to set at the start of the session, and what how to import the libraries

  - Camera Connect – explains how to connect to the cameras, and shows how a camera_config file is accessed and used to load and setup the cameras.

  - Spectralon Imaging – explains how setup the scene labels, check and choose the ROI manually, autoexpose the images, and then capture and save the data

  - Sample Imaging – follows the same procedure as above, with a few variable names changed.

  - Dark Frame imaging – shows how to change the scene and variables

# Key Functions

Autoexposure – follows algorithm of Maki et al 2003: §3.1.4 https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2003JE002077

Key parameters are:

- Init_t_exp: initial exposure. If set to ‘CURRENT’ it uses whatever the current camera exposure is to seed the algorithm.

- Target: either int in range (1, 4096) giving the target DN number, or float (0, 0.99) giving the fraction of the sensor range, for the target

- n_hot: the number of pixels allowed to exceed the target level. Either int, giving number of pixels, or float in range (0, 0.99), giving the fraction of the image region of interest allowed to exceed the Target.

- ROI: if true, auto-exposure over the ROI that has been set for the camera. If false, use the entire frame.

- Tol: the absolute tolerance threshold. How close the image statistic needs to be to the target before the exposure is accepted.

- Limit: int, the number of iterations of the algorithm before it is terminated.